Name – Mayank Trivedi

Module – 2: A+ - Peripherals And Power Supply

Section 1: Multiple Choice

- Which of the following precautions should be taken before working on computer hardware?

- Ensure the computer is plugged in to prevent electrostatic

Discharge.

- Wear an anti-static wrist strap to prevent damage from

electrostatic discharge.

- Work on carpeted surfaces to prevent slipping.

- Use magnetic tools to handle components more easily.

Ans. Wear an anti-static wrist strap to prevent damage from

electrostatic discharge.

- What is the purpose of thermal paste during CPU installation?

- To insulate the CPU from heat.

- To provide mechanical support for the CPU.

- To improve thermal conductivity between the CPU and the heat sink.

- To prevent the CPU from overheating.

Ans. To improve thermal conductivity between the CPU and the heat sink.

- Which tool is used to measure the output voltage of a power

supply unit (PSU)?

- Multimeter

- Screwdriver

- Pliers

- Hex key

Ans. Multimeter

- Which component is responsible for storing BIOS settings, such as date and time, even when the computer is powered off?

- CMOS battery

- CPU

- RAM

- Hard drive

Ans.CMOS battery

Section 2: True or False

- True or False: When installing a new hard drive, it is essential to

format it before use.

Ans.True

- True or False: A POST (Power-On Self-Test) error indicates a

problem with the CPU.

Ans.False

- True or False: It is safe to remove a USB flash drive from a

computer without ejecting it first.

Ans.False

Section 3: Short Answer

- Describe the steps involved in installing a new graphics card in a

desktop computer.

Ans. Steps involved in installing a new graphics card are :

- Turn off the computer and unplug it

- Open the computer case.

- Locate the PCI Express (PCIe) x16 slot on the motherboard.

- Remove the existing graphics card (if any).

- Insert the new graphics card firmly into the PCIe slot.

- Secure the card with screws.

- Connect the power cable (if required).

- Close the case, plug in the computer, and install the latest drivers.

- What is RAID, and what are some common RAID configurations?

Ans. RAID (Redundant Array of Independent Disks) is a

technology that combines multiple hard drives to improve

performance, reliability, or both.

Common RAID configurations:

- RAID 0: Data is split across drives for speed (no data protection).

- RAID 1: Data is mirrored (copied) on two drives for redundancy.

- RAID 5: Data and parity distributed across 3+ drives (balanced speed and protection).

- RAID 10 (1+0): Combines mirroring and striping for both speed and redundancy.

RAID helps in improving data access speed or data safety. Depending on the configuration, it can provide faster performance, fault tolerance, or both.

Section 4: Practical Application

- Demonstrate how to replace a CPU fan in a desktop computer.

Ans. Steps to replace a CPU fan:

- Turn off the computer and unplug it.

- Open the computer case.

- Disconnect the old fan’s power cable from the motherboard.

- Unscrew or unclip the old fan and remove it carefully.

- Clean the CPU surface and apply a small amount of thermal paste.

- Place the new fan (or cooler) on top of the CPU and secure it with screws or clips.

- Connect the new fan’s power cable to the CPU_FAN header on the motherboard.

- Close the case and power on the computer to test.

Section 5: Essay

- Discuss the importance of regular maintenance for computer

hardware and provide examples of maintenance tasks.

Ans. Regular maintenance of computer hardware is one of the most important practices to ensure that a computer system runs efficiently, remains reliable, and lasts longer. Computers, like any other machines, are made up of mechanical and electronic components that degrade over time due to dust, heat, and continuous use. Performing regular maintenance helps prevent major issues, enhances performance, and reduces repair costs.

1. Improves Overall Performance
Over time, computers accumulate dust and unnecessary files that slow down system performance. Regular cleaning of internal components such as fans, CPU heatsinks, and RAM slots ensures proper ventilation and prevents overheating. Routine software maintenance like clearing cache, deleting temporary files, and optimizing startup programs also keeps the system running smoothly.

2. Prevents Hardware Failures
Regular inspections can help detect loose connections, failing fans, or power supply problems before they cause complete hardware failure. Early detection of such issues can prevent costly damage to the motherboard, hard drives, or graphics card, which could otherwise lead to system crashes or data loss.

3. Extends Hardware Lifespan
Just like regular servicing extends the life of a vehicle, maintaining computer hardware helps extend the lifespan of components. Keeping the internal temperature under control, ensuring stable power input, and avoiding physical wear and tear all contribute to making devices last longer and function efficiently.

4. Enhances Data Safety
Hardware issues such as failing hard drives or power surges can cause severe data loss. Regular maintenance includes checking the health of storage devices, creating backups, and using surge protectors. These steps safeguard important data and prevent loss due to hardware malfunction.

5. Ensures System Reliability
Regular maintenance ensures that the system remains stable and reliable during important tasks. A well-maintained system is less likely to freeze, crash, or lag, which improves productivity, especially in workplaces where downtime can affect business operations.

6. Saves Cost and Time
Preventive maintenance is far more cost-effective than emergency repairs. Replacing a small fan or cleaning a clogged vent is cheaper than replacing a damaged motherboard or CPU. Additionally, timely maintenance reduces downtime and helps users avoid losing valuable work hours due to unexpected failures.

7. Keeps the System Secure
Hardware maintenance often goes hand in hand with software updates. Ensuring that the BIOS, drivers, and firmware are up to date helps close security loopholes that hackers can exploit. It also ensures compatibility with newer operating systems and applications.

8. Improves Energy Efficiency
A clean and well-maintained system consumes less power. When cooling fans and power units work efficiently, the system generates less heat and runs more effectively, reducing electricity usage and contributing to overall energy savings.

Examples of Maintenance Tasks:
- Cleaning dust from CPU fans, heat sinks, and vents.
- Checking and reseating cables and RAM modules.
- Updating drivers, BIOS, and firmware.
- Running antivirus and malware scans regularly.
- Performing disk cleanup and defragmentation.
- Monitoring hard drive health using diagnostic tools.
- Ensuring proper ventilation and cooling in the system.
- Backing up important files and data.

Conclusion
In conclusion, regular maintenance of computer hardware is not just a routine task but a necessity for ensuring smooth performance, reliability, and data safety. It prevents major hardware failures, increases the lifespan of components, and saves money in the long run. By investing a little time in preventive maintenance, users can enjoy faster, safer, and more efficient computer systems for years.